Załącznik 1 Konfiguracja AP AP3915i-ROW

| Fully-Featured Enterprise Class AP | / |
| --- | --- |
| Number of Wi-Fi Radios | 2 |
| MIMO Implementation for High-Performance 11ac & 11n Throughputs | 2x2 |
| Number of Spatial Streams | 2 |
| Number of Simultaneous Users (MU-MIMO) | 2 |
| Maximum Throughput 2.4GHz Radio | 300 Mbps |
| Maximum Throughput 5GHz Radio | 867 Mbps |
| Maximum Throughput Per AP | 1.167 Gbps |
| RFC2285 Wire/Wireless Forwarding Rate | 50,000 pps |
| Number of SSIDs Supported Per Radio/Total | 8/16 |
| Simultaneous Users Per Radio/Total | 240/480 Per AP |
| Simultaneous Voice calls (802.11b, G711, R>80) | 30 or less |
| Mode of Operation | Semi-autonomous |
| Plug and Play Operation/Zero Touch Deployment | / |
| Security and Standards | WPA, WPA2 (AES), 802.11i, 802.1x, IPSec, IKEv2, PKCS #10, X509 DER/ PKCS #12, SSL |
| Internet of Things (IoT) Radio | Dual mode selectable (2.4 GHz with Co-Existence): Bluetooth Low Energy (BLE) 4.1 Single and Dual mode operation (Classic and Low Power Profiles 802.15.4 -2011) |
| Multiple Operating Modes |  |
| Intelligent Thin AP | Encryption, Security, QoS and RF Management Services done at AP |
| Distributed and Centralized Data Paths Within Same SSID | / |
| Application Based Distributed and Centralized Data Paths Within Same User/ Device Session | / |
| Simultaneous RF Monitoring and Client Services | / |
| BYOD / Device Fingerprinting Visibility | / |
| Application / Layer 7 Visibility and Control | / |
| In-Channel WIDS | / |
| In-Channel WIPS | / |
| Dedicated Multi-Channel WIDS (Guardian Mode) | / |
| Dedicated Multi-Channel WIPS (Guardian mode) | / |
| Dedicated Multi-Channel RF Spectrum Analysis and Fingerprinting | / |
| Locates Devices and Threats via RF Triangulation | / |
| Self-Forming and Self-Healing Meshing | / |

| Remote Access Point | / |
| --- | --- |
| Hardware-Based, End-to-End Data and Control Plane Encryption | / |
| Private and Public Cloud Deployments | / |
| SSL | / |
| Policy Enforcement for Wireless Clients (L2-L7 Access Control, QoS, Rate Limiting, and VLAN Containment) | / |
| Fabric Attach ready (end-to-end network orchestration and automation) / |  |
| Hybrid Options |  |
| Security Scanning and Serve Clients On Same Radio | / |
| Security Scanning and Spectrum Analysis On Same Radio | / |
| Spectrum Analysis and Serve Clients On Same Radio | / |
| Multi-Channel Dedicated Security Scanning and Spectrum Analysis | / |
| Radio Characteristics |  |
| Cellular coexistence | / |
| Max Total Conducted Power (does not include antenna gain*) |  |
| Radio 1 (5GHz) | 26 dBm |
| Radio 2 (2.4GHz) | 26 dBm |
| IoT Radio | 3 dBm |
| Max Antenna Gain (Integrated Antenna) |  |
| Radio 1 (5GHz) | 5.3 dBi |
| Radio 2 (2.4GHz) | 4.3 dBi |
| IoT | 4.1 dBi |
| Adaptive Radio Management |  |
| Dynamic Channel Control | 802.11h: DFS and TPC support (ETSI) |
| Efficient Use of the Spectrum with A Multi-Channel Architecture | / |
| Automatic Transmit Power and Channel Control | / |
| Self-Healing with Coverage Gap Detection | / |
| Band Steering with Multiple Steering Modes | / |
| Spectrum Load Balancing of Clients | / |
| Airtime Fairness | / |
| Performance Protection In Congested RF Environments | / |
| Fast Transition Roaming (802.11k) | / |
| Mitigates Co-Channel Interference with Coordinated Access | / |
| Mitigates Adjacent Channel Interference with Optimized Receive Sensitivity | / |
| Efficient Reuse of Channels At Shorter Intervals | / |
| Mitigates Non 802.11 Interference Without Dedicated Radios | / |
| Probe Suppression and Client Link Monitoring | / |
| Management Frame Protection (802.11w) | / |
| Quality of Service |  |
| Quality of Service (WMM, 802.11e) | / |
| Power Save (U-APSD) | / |
| Fast Secure Roaming and Handover Between APs (802.11r) | / |
| Pre-Authentication (Pre-Auth) / |  |

| Opportunistic Key Caching (OKC) | / |
| --- | --- |
| Bonjour/LLMNR/UPNP Identification, Containment and Control | / |
| Supports Voice, Video, and Data Using the Same SSID | / |
| Prioritizes Voice Over Data for Both Tagged and Untagged Traffic | / |
| Rate Limiting (Rule and User-Based) | / |
| Rule and Role Based Qos Processing | / |
| Multicast Rate Control |  |
| Multicast to Unicast Conversion / |  |
| Adaptable Rate Multicast | / |
| Power Save Mode Optimization for Multicast | / |
| Media Access Protocol | CSMA/CA with ACK |
| Data Rates | 802.11a: 6, 9, 12, 18, 24, 36, 48, 54 Mbps 802.11b:1, 2, 5.5,11 Mbps 802.11g: 1, 2, 5.5, 6, 9,11, 12, 18, 24, 36, 48, 54 Mbps 802.11n: Performance Table below 802.11ac: Performance Table below Receiver Sensitivity 802.11a: • -94dBm@6Mbps • -79dBm @ 54Mbps 802.11g: • -95dBm@6Mbps • -79dBm @ 54Mbps 802.11n: See 802.11n Receiver Sensitivity Table below 802.11ac: See 802.11ac Receiver Sensitivity Table below |
| Frequency Bands | 802.11 ac/a/n: • 5.15 to 5.25 GHz (FCC/IC/ETSI) • 5.25 to 5.35 GHz (FCC/IC/ETSI)* • 5.47 to 5.725 GHz (FCC/IC/ETSI)* • 5.725 to 5.850 GHz (FCC/IC) 802.11b/g/n: • 2.400 to 2.4720 GHz (FCC/IC) • 2.400 to 2.4835 GHz (ETSI) *FCC/IC DFS planned after release |
| Wireless Modulation | 802.11ac: BPSK, QPSK, 16QAM, 64QAM, 256QAM with OFDM 802.11ac Packet Aggregation: A-MPDU, A-MSDU 802.11ac Very High-Throughput (VHT): VHT20/40/80 802.11ac Advanced Features: LDPC, STBC, Maximum Likelihood (ML) Detection 802.11n: BPSK, QPSK, 16QAM, 64QAM with OFDM 802.11n High-throughput (HT) support: HT 20/40 802.11n Packet aggregation: A-MPDU, A-MSDU 802.11n Advanced Features: LDPC, STBC and TxBF 802.11a: BPSK, QPSK, 16QAM, 64QAM with OFDM 802.11g: DSSS and OFDM 802.11b: DSSS |
| Interfaces |  |
| Uplink | 1x 10/100/1000 Mbps auto-sensing Ethernet port |
| Console Port | RJ45 |
| Ext. 12VDC | / |

| Wireless and EMC |  |
| --- | --- |
| Compliance | • FCC CFR 47 Part 15, Class B • ICES-003 Class B • FCC SubpartC 15.247 • FCC Subpart E 15.407 • RSS-247 • EN 301 893 • EN 300 328 • EN301489 1&17 • EN50385 • EN 55032 (CISPR 32) • EN 60601-1-2 • AS/NZS4268 + CISPR32 |
| Safety | • IEC 60950-1 • EN 60950-1 • UL 60950-1 • CSA22.2 No.60950-1-03 • AS/NZS 60950.1 |
| Mechanical |  |
| Dimensions | 6.34 in. diameter x 1.74 in. height /161 mm. x 48.5mm 6.89 in. |